riscv64-unknown-elf-gcc -O1 -mabi=lp64 -march=rv64i -o sum_1ton.o sum_1ton.c

riscv64-unknown-elf-gcc -O1 -mabi=lp64 -march=rv64i -o sum_1ton.o sum_1ton.c

riscv64-unknown-elf-objdump -d sum_1ton.o

#include <stdio.h>

// Function to convert binary to Gray code

int binary_to_gray(int num) {

return num ^ (num >> 1);

}

int main() {

int binary, gray;

// Input a binary number

printf("Enter a binary number: ");

scanf("%d", &binary);

// Convert binary to Gray code

gray = binary_to_gray(binary);

// Output the Gray code

printf("Gray code = %d\n", gray);

return 0;

}

Explanation:

- Opcode (7 bits - 0110011): Indicates that this is an R-type instruction.

- rd (5 bits - 01011): Specifies that the result will be stored in a1 (x11).

- funct3 (3 bits - 100): Specifies that this is a bitwise XOR operation.

- rs1 (5 bits - 01011): Specifies that the first operand is a1 (x11).

- rs2 (5 bits - 01111): Specifies that the second operand is a5 (x15).

- funct7 (7 bits - 0000000): Differentiates the XOR operation.

This instruction tells the processor to perform a bitwise XOR between the values in a1 (register x11) and a5 (register x15), and then store the result back in a1 (register x11).

Xor a1,a1,a5

Explanation:

- Opcode (0110011): Specifies that this is an R-type instruction.

- funct7 (0000000): Indicates a logical operation (and differentiates XOR from similar instructions).

- rs2 (01111): Specifies the second source register, which is x15 (a5).

- rs1 (01011): Specifies the first source register, which is x11 (a1).

- funct3 (100): Indicates the XOR operation.

- rd (01011): Specifies the destination register, which is x11 (a1).

Operation:

This instruction performs a bitwise XOR between the contents of the a1 and a5 registers (i.e., x11 and x15), and the result is stored back into the a1 register (i.e., x11).

Example:

- Before Execution:

  - a1 = 0b1010 (decimal 10)

  - a5 = 0b1100 (decimal 12)

- After Execution:

  - a1 = 0b0110 (decimal 6), which is the result of 0b1010 XOR 0b1100.

Xpr a1,a1,a5

Lui a0,0x2b

module riscv_processor (

input wire clk,

input wire reset,

// Other inputs/outputs

);

// Register file

reg [31:0] registers [0:31];

// Instruction register

reg [31:0] instruction;

// Control signals

wire [6:0] opcode;

wire [4:0] rs1, rs2, rd;

wire [2:0] funct3;

wire [6:0] funct7;

// Extract instruction fields

assign {funct7, rs2, rs1, funct3, rd, opcode} = instruction;

// Opcode for XOR instruction

localparam OPCODE_R_TYPE = 7'b0110011;

localparam FUNCT3_XOR = 3'b100;

localparam FUNCT7_XOR = 7'b0000000;

// Main execution block

always @(posedge clk or posedge reset) begin

if (reset) begin

// Initialize registers or handle reset logic

end else begin

// Instruction decoding and execution

if (opcode == OPCODE_R_TYPE) begin

case (funct3)

FUNCT3_XOR: begin

if (funct7 == FUNCT7_XOR) begin

// Perform XOR operation

registers[rd] <= registers[rs1] ^ registers[rs2];

end

end

// Other R-type instructions can be added here

endcase

end

end

end

// Instruction fetch, decode, and other stages should be added here

endmodule